Technical Specifications Veggie Car Imagine cruising the highways and byways in a sports car powered by biodiesel getting 30 miles per gallon. That’s what I do with the Veggie Car – a 1971 Datsun 240Z powered by a Nissan LD28 2800 cc displacement diesel engine with approximately 93 horsepower. Since the 240z did not come with a diesel, I got a diesel engine from a wrecked 1982 Nissan Maxima. (The LD28 engine is a popular engine in Europe, Africa, Australia and Japan. In the US, it was only available in a few Maximas, some forklifts and some boats). I only made two modifications to the LD28 Nissan diesel in-line 6-cylinder engine before installing it. I modified the oil pan to fit over the 240z’s front suspension and I replaced the 32-year old rubber fuel hose that goes to the fuel tank with a new Viton-brand synthetic fuel hose. The engine dropped straight in, mounted to the original mounting brackets and mated with the original transmission, which I have since replaced with a 1978 5-speed tranny. Although biodiesel studies done show anywhere from a 0% to a 10% loss in power, I notice no power loss with 100% biodiesel (B100) in the Veggie Car - which easily accelerates to over 100 mph. My problem is not power loss – it’s speeding tickets, of which I now have a small collection. Doug at Fairlady Z Motors in Portland, OR will soon outfit the Veggie Car with a custom Turbocharger made. The turbocharger will increase horsepower by 25% - bringing the LD28 up to the same performance as the original gasoline L24 that came standard in the 1971 240z.; LD 28 Engine Specs. Manufacturer; Nissan. Year; 1983. Model; LD28. Displacement; 170.35 cu. in. (2792cc). Bore x Stroke:; 3.327x3.268in. (84.5 x 83mm). Type; 4-cycle, OHC. Compression Ratio:; 22.0 : 1. Combustion:; Swirl Chamber. Cylinders:; 6, in-line. Horsepower*; Approx. 93@4400 rpm without Turbo approx. 116@4400 rpm with Turbo. Torque; Approx. 148 lb-ft @ 2000 rpm without Turbo. Transmission; 5 spd manual. Weight (Dry):; Approx. 415 lbs (238 kg). * The horsepower of the diesel without the turbo is estimated to be approx. 75% of its gas counterpart...